AZ549G

10GE XGS-PON SFU

Descriptions

The AZ549G is a 10GE XGSPON fiber optic modem, which is designed to meet the FTTH ultra-wide band access requirements of home and SOHO users. It supports NAT / firewall and other functions. It is based on stable and mature GPON technology with high cost-performance and layer 2 Ethernet switch technology. It is reliable and easy to maintain, guarantees QoS, and fully conforms to ITU-T g.9807 XGSPON standard.

Features

●   XGS-PON WAN port with 10Gbps uplink / 10Gbps downlink link speed;

●   1x 10/100/1000/2.5G/5G/10G BASE-T Ethernet RJ45 Ports;

Specifications

| AZ549G Specifications |  |
| --- | --- |
| CPU | Dual-Core ARM Cortex-A55 at 1.0GHz |
| Chip model | RTL9615C |
| Memory | NAND FLASH 1Gbit(128MB)/DDR4 4Gbit（512MB） |
| Bob Driver | GN28L98 |
| PON Protocol Specification | The XGS-PON(10G/10G)ITU-T G.9807.1 MAC supports the following features Supports data rate of: ~10Gbps(9953 Mbps)downstream. ~10Gbps(9953Mbps) upstream. Up to 256 GEM port-IDs. Up to 40 AlloclDs. Ranging and activation process support. Downstream characteristics: -Downstream AES-128 encryption and key exchange process. -Downstream FECRS(248216). -Dedicated independent queues for PLOAM and CPU packets. Upstream characteristics: -Upstream AES-128 encryption and key exchange process. -Upstream FECRS(248216.) -Normal queues for PLOAM and CPU. Support ToD synchronization perITU-TG.9807.1. Support ONU power saving operation perITU-TG.9807.1. |
| Network Protocol Specifications | ANSI/IEEE 802.3 NWay auto-negotiation 802.1D bridging VLAN Translation IP ToS/DSCP to 802.1p mapping Class of Service based on UNI, VLAN-ID, 802.1p bit, and combination Marking/remarking of 802.1p IGMP v2/v3 snooping and IGMP snooping with proxy report Broadcast/Multicast rate limiting |

|  | 802.1Q VLAN tagging/un-tagging Support flexible traffic classification Support VLAN Intelligent Bridging and Cross Connect mode |
| --- | --- |
| Interface | WAN: 1*10G optical interface (APC) LAN: 1*10/100/1000/2.5G/5G/10G auto MDI/MDI-X RJ-45 ports |
| LED Indicators | Power, PON, LOS, INTERNET, 10G LAN |
| Power Supply | DC12V 1.0A |
| Security | Firewall, Dos Protection |
| WAN Networking | Static IP WAN connection DHCP client WAN connection PPPoE WAN connection |
| Management | Standard OMCI (G.988) Web GUI (HTTP/HTTPS) Firmware upgrade via FTP/TFTP/HTTP Telnet, Web, SSH CLI command via telnet/console Two-level administration privilege for web configuration Configuration backup/restore TR069 |
| Product Size | 130X130X36.5mm(length X width X height) |
| Work Environment | Working temperature: -0°C—40°C Working humidity: 5—95% |

Applications

Contact us

AZROAD TECHNOLOGY COMPANY LTD.

Mail to: sales@azroad.com

URL: http://www.azroad.com/en

Specifications are subject to change without notice.

Copyright © AZROAD. All rights reserved.