AZ546G-S

4GE AX3000 GPON HGU

Descriptions

AZ546G-S GPON ONT is a new terminal level GPON ONT, which is a variety of FTTB \ FTTH \ FTTO access ONT

manufactured and supplied by AZROAD for carrier-grade FTTH network applications. It can support pure data,

WIFI and many other services accessing type. AZ546G-S ONT supports standard OMCI, which provides up to

1.25Gbps uplink and 2.5Gbps downlink high-speed transfer rate. AZ546G-S supports 802.11ax dual band Wi-Fi

speed up to 2.4Gbps.

Features

●   GPON WAN port with 1.244Gbps uplink / 2.488Gbps downlink link speed;

●   4x 10/100/1000BASE-T Ethernet RJ45 Ports;

●   Support 802.11n/ax 2T2R speed up to 574Mbps;

●   Support 802.11ac/ax 2T3R speed up to 2400Mbps;

●   Support Voip;

●   Support USB2.0

Specifications

| AZ546G-S Specifications |  |
| --- | --- |
| CPU | 1.0GHz ARM CA53 dual-core CPU,256K L2 cache |
| Chip model | EN7529CU+MT7916AN+MT7976DN+EN7571N+Le9643 |
| Memory | NAND FLASH 1Gbit(128MB)/ Integrate 512MB DDR3 SDRAM |
| WiFi Chip | 2.4G&5G: MT7916AN+MT7976DN |
| Bob Driver | EN7571N |
| GPON Protocol Specification | Comply ITU-T G.984 GPON standard: G.984.1 general characteristics G.984.2 physical Media Dependent (PMD) layer specifications G.984.3 transmission convergence layer specifications G.984.4 ONT management and control interface specification Support DS/US transmission rate to 2.488 Gbps/1.244 Gbps Wavelength: 1490 nm downstream & 1310 nm upstream Comply with class B+ type PMD Physical distance reach to 20 km Support Dynamic Bandwidth Allocation (DBA) GPON Encapsulation Method (GEM) supports Ethernet packet Supports GEM header removal/insertion and data extraction/segmentation (GEM SAR) Configurable AES DS and FEC DS/US Support up to 8 T-CONs each with priority queues (US) |
| Network Protocol Specifications | 802.3 10/100/1000 Base T Ethernet ANSI/IEEE 802.3 NWay auto-negotiation 802.1Q VLAN tagging/un-tagging Support flexible traffic classification Support VLAN staking Support VLAN Intelligent Bridging and Cross Connect mode |

| Interface | WAN: One Giga optical interface (APC or UPC) LAN: 4*10/100/1000 auto MDI/MDI-X RJ-45 ports Voip: 1*RJ-11 port USB: 1* USB2.0 |
| --- | --- |
| LED Indicators | Power, PON/LOS, INT, LAN4-1, 2.4G WLAN, 5G WLAN , WPS, USB, TEL |
| WiFi Rate | 2.4G: 802.11n/ax with 574Mbps 5G: 802.11ac/ax with 2400Mbps |
| Frequency Band | 2.4 GHz frequency band 40MHz and 5.8 GHz frequency band 160MHz |
| Buttons | WPS/WLAN/Reset |
| Antennas | 1*5G 5dBi Internal antennas 2*2.4G&5G 4dBi External antennas |
| Power Supply | DC12V 1.5A |
| Product Size | 211.5X151.9X38.5 mm (length X width X height) |
| Work Environment | Working temperature: 0°C—40°C Working humidity: 5—95% |
| Security | WPA/WPA2/WPA3, 64/128bit WEP data encryption Firewall, Dos Protection, ssl connection |
| WAN Networking | Static IP WAN connection DHCP client WAN connection PPPoE WAN connection |
| VOIP | G.711A/μ, G.729, and G.722 encoding/decoding T.30/T.38/G.711 fax mode DTMF /Emergency calls (with the SIP protocol) |
| Management | Standard OMCI (G.984.4) Web GUI (HTTP/HTTPS) Firmware upgrade via TFTP/HTTP/TR069/OMCI Telnet, Web, SSH CLI command via telnet/console Configuration backup/restore TR069 |
| Certification | Meet CE certification |