AZ554G

5GE BE3600 GPON HGU

Descriptions

AZ554G GPON ONT is a new terminal level GPON ONT, which is a variety of FTTB \ FTTH \ FTTO access ONT

manufactured and supplied by AZROAD for carrier-grade FTTH network applications. It can support pure data,

WIFI and many other services accessing type. AZ554G ONT supports standard OMCI, which provides up to

1.25Gbps uplink and 2.5Gbps downlink high-speed transfer rate. AZ554G supports 802.11be dual band Wi-Fi

speed up to 3.6Gbps.

Features

●   GPON WAN port with 1.244Gbps uplink / 2.488Gbps downlink link speed;

●   1x 10/100/1000/2500BASE-T Ethernet RJ45 Ports

●   4x 10/100/1000BASE-T Ethernet RJ45 Ports;

●   Support 802.11be 2T2R speed up to 689Mbps;

●   Support 802.11be 2T3R speed up to 2882Mbps;

●   Support Voip;

●   Support USB2.0

Specifications

| AZ554G Specifications |  |
| --- | --- |
| CPU | 1.2GHz ARM CA53 dual-core |
| Chip model | AN7553CT+ MT7990AN+MT7976CN+ AN8801SBN+ EN7571N+Le9643 |
| Memory | NAND FLASH 2Gbit(256MB)/ 512MB DDR3 SDRAM |
| Wi-Fi Chip | MT7990AN+MT7976CN |
| Bob Driver | EN7571N |
| GPON Protocol Specification | Comply ITU-T G.984 GPON standard: G.984.1 general characteristics G.984.2 physical Media Dependent (PMD) layer specifications G.984.3 transmission convergence layer specifications G.984.4 ONT management and control interface specification Support DS/US transmission rate to 2.488 Gbps/1.244 Gbps Wavelength: 1490 nm downstream & 1310 nm upstream Comply with class B+ type PMD Physical distance reach to 20 km Support Dynamic Bandwidth Allocation (DBA) GPON Encapsulation Method (GEM) supports Ethernet packet Supports GEM header removal/insertion and data extraction/segmentation (GEM SAR) Configurable AES DS and FEC DS/US Support up to 8 T-CONs each with priority queues (US) |
| Network Protocol Specifications | 802.3 10/100/1000 Base T Ethernet ANSI/IEEE 802.3 NWay auto-negotiation 802.1Q VLAN tagging/un-tagging Support flexible traffic classification Support VLAN staking Support VLAN Intelligent Bridging and Cross Connect mode |

| Interface | WAN: One Giga optical interface (APC or UPC) LAN: 1*10/100/1000Mbps/2500Mbps+4*10/100/1000 auto MDI/MDI-X RJ-45 ports Voip: 1*RJ-11 port USB: 1*USB2.0 |
| --- | --- |
| LED Indicators | Power, INT, PON/LOS, 2.5G LAN. LAN4-1, 2.4G WLAN, 5G WLAN, WPS, TEL, USB |
| Wi-Fi Rate | 2.4G: 802.11be with 689Mbps 5G: 802.11be with 2882Mbps |
| Frequency Band | 2.4 GHz frequency band 40MHz and 5.8 GHz frequency band 160MHz |
| Buttons | WPS/WLAN/Reset |
| Antennas | 1*5G 3dBi Internal antennas 2*2.4G&5G 4dBi External antennas |
| Power Supply | DC12V 1.5A |
| Product Size | 215X130X43 mm (length X width X height) |
| Work Environment | Working temperature: 0°C—40°C Working humidity: 5—95% |
| Security | WPA/WPA2/WPA3, 64/128bit WEP data encryption Firewall, Dos Protection, ssl connection |
| WAN Networking | Static IP WAN connection DHCP client WAN connection PPPoE WAN connection |
| VOIP | G.711A/μ, G.729, and G.722 encoding/decoding T.30/T.38/G.711 fax mode DTMF /Emergency calls (with the SIP protocol) |
| Management | Standard OMCI (G.984.4) Web GUI (HTTP/HTTPS) Firmware upgrade via TFTP/HTTP/TR069/OMCI Telnet, Web, SSH CLI command via telnet/console Configuration backup/restore TR069 |
| Certification | Meet CE certification |