AZ638

AX3000 VDSL IAD

Descriptions

The AZ638 is a Wi-Fi 6 Gigabit dual band wireless IAD, which is upgraded to the 802.11ax technical standard and integrates VDSL/VoIP/wireless AP. It is a security solution for home, broadband remote office and small-scale SOHO network.

Features

●   DSL WAN port with 50Mbps uplink / 350Mbps downlink link speed ;

●   4x 10/100/1000BASE-T Ethernet RJ45 Ports ;

●   Support 802.11n/ax 2T2R speed up to 574Mbps ;

●   Support 802.11ac/ax 3T3R speed up to 2402Mbps ;

●   Support one FXS ports ;

●   Support one USB2.0 port ;

Specifications

| AZ638 Specifications |  |
| --- | --- |
| CPU | 1GHz ARM CA53 dual-core 256K L2 cache |
| Chip model | EN7562CT+MT7916AN+MT7976DN+EN7556N+EN7517BU+Le9643 |
| Memory | NAND FLASH 1Gbit(128MB)/DDR3 2Gbit（512MB） |
| Wi-Fi Chip | 2.4G&5G: MT7916AN+MT7976DN |
| DSL Driver | EN7556N |
| DSL Protocol Specification | G.992.1 (G.dmt), G.992.2 (G.lite), G.992.3 (ADSL2), G.992.5 (ADSL2+) Annex A/B/I/J/M/L ADSL physical connection ATM AAL5 RFC 2684/1483 G.993.2 VDSL2 (Up to 35b profile) Band plan: Annex A/B Dual latency UPBO/DPBO Double-end line test (DELT) Auto SRA Bit Swap |
| Network Protocols | IPv4/IPv6 Dual Stack TCP/UDP/ICMP HTTP/HTTPS/FTP/TFTP/TELNET/SSH VLAN NAT/PAT/ALG UPnP IGD with NAT traversal capability RIP v1/v2 DNS Proxy/Dynamic DNS SIP/RTP DHCP client/server/relay IGMP and MLD snooping/proxy |

| Interface | WAN: RJ-11 Port with 35b LAN: 4*10/100/1000 auto MDI/MDI-X RJ-45 ports VoIP: RJ-11 port USB: 1*USB2.0 |
| --- | --- |
| Buttons | WPS/Reset/Wi-Fi/Power on/off |
| LED Indicators | Power, DSL, Internet, LAN4-LAN1, 2.4GWIFI, 5GWIFI, WPS, USB, FXS |
| Wi-Fi Rate | 2.4G: 802.11b/g/n/ax@574Mbps 5G: 802.11/a/n/ac/ax@2402Mbps |
| Frequency Band | 2.4 GHz frequency band 40MHz and 5.8 GHz frequency band 160MHz |
| Antennas | 2*3dBi External dual-band omni antennas 1*3dBi built-in 5G omni antenna |
| Power Supply | DC12V 2A（Without USB 1.5A） |
| Security | Stateful Packet Inspection (SPI) Denial of Service (DoS) prevention Multiple VPN (IPSec, PPTP, L2TP) passthrough MAC/IP/URL filtering Black and white list Port mapping/DMZ/Virtual server Remote Access Control |
| WAN Networking | Static IP WAN connection DHCP client WAN connection PPPoE WAN connection 3G/4G dongle VPN client/passthrough Router/Bridge Mode |
| VOIP | G.711A/μ, G.729, and G.722 encoding/decoding T.30/T.38/G.711 fax mode DTMF /Emergency calls (with the SIP protocol) |
| Wireless | Basic/Advanced Settings WEP/WPA1/WPA2/WPA3/WPS MESH Band Steering Multiple SSID WMM Channel scan WDS ACL list |
| Management | TR-069 remote management Web GUI management Local built-in diagnostic function Logs and statistics Remote upgrade by HTTP Saving/Restoring configuration to/from file SNTP and time zone Multi-language |
| Product Size | 211.5X151.9X38.5mm (length x width x height) |
| Work Environment | Working temperature: 0°C—40°C Working humidity: 5—95% |
| Certification | Meet CE certification |