AZ7005

AX3000 WiFi Router

Descriptions

AZ7005 is a concurrent dual band ax3000 wireless router, which has been upgraded to the latest 802.11ax standard

technology while maintaining compatibility with IEEE 802.11b/g/n and IEEE 802.11a networks. It’s a SOHO-class

network security solution for family home, broadband communications and small SOHO.

Features

●   1x 10/100/1000BASE-T Ethernet RJ45 WAN Port;

●   3x 10/100/1000BASE-T Ethernet RJ45 LAN Ports;

●   WAN port support passive POE out for ODU（option）;

●   Support one VOIP port;

●   Support 802.11ax 2T2R speed up to 574Mbps;

●   Support 802.11ax 2T2R speed up to 2402Mbps;

●   Four Antennas for good coverage;

Specifications

| AZ7005 Specifications |  |
| --- | --- |
| CPU | ARM CA53 (1GHz, Dual-core) |
| Chip model | EN7562CT+MT7916AN+MT7976DN+Le9643 |
| Memory | NAND FLASH 1Gbit(128MB)/DDR3 4Gbit（512MB） |
| WiFi Chip | 2.4G&5G: MT7916AN+MT7976DN |
| Interface | WAN: 1* 10/100/1000Mbps auto MDI/MDI-X RJ-45 port (passive POE out) LAN: 3* 10/100/1000Mbps auto MDI/MDI-X RJ-45 ports 1* VOIP port |
| Buttons | WiFi/WPS, Reset |
| LED Indicators | POWER, Internet, LAN3-LAN1, WAN,2.4G,5G, WPS, FXS |
| WiFi Rate | 2.4G:802.11b/g/n/ax@574Mbps 5G:802.11/a/an/ac/ax@2402Mbps |
| Frequency Band | 2.4 GHz frequency band 40MHz and 5 GHz frequency band 160MHz |
| Antennas | 2*5G 5dBi External antennas 1*2.4G 5dBi External antenna 1*2.4&5G 3dBi Dual-band External antenna |
| Power Supply | DC 54V/1A or DC12V/1.5A (no support POE) |
| Product Size | 180X120X42.2 mm (length X width X height) |
| Work Environment | Working temperature: 0°C—40°C Working humidity: 5—95% |
| WAN Connection | Support DHCP, PPPoE and Static IP address three internet access models, Support WAN automatic detection, MAC Clone |
| Wireless Function | WiFi5/WiFi6 basic function, Guest network, Intelligent networking, Wireless Repeater， Easy mesh |
| Wireless Encryption | WPA/WPA2, WPA2, WPA3 |
| Management Function | WEB, Telnet/CLI |
| System Maintenance | Device management, Configuration management, Upgrade management, Log function, Status statistics, Restart/reset |
| Advanced Function | Parental control, Virtual server, DDNS, VPN, IPV6, UPnP |
| Certification | Meet CE certification |