AZ7006

BE3600 Router

Descriptions

AZ7006 is a concurrent dual band BE3600 ROUTER, which has been upgraded to the latest 802.11be standard

technology while maintaining compatibility with IEEE 802.11b/g/n/ax and IEEE 802.11a/an/ac/ax networks. It’s a SOHO-class

network security solution for family home, broadband communications and small SOHO.

Features

●   1x 10/100/1000/2500 BASE-T Ethernet RJ45 WAN Ports ;

●   3x 10/100/1000 BASE-T Ethernet RJ45 LAN Ports ;

●   Support 802.11be 2T2R speed up to 689Mbps ;

●   Support 802.11be 2T2R speed up to 2882Mbps ;

●   Five Antennas for good coverage ;

Specifications

| AZ7006 Specifications |  |
| --- | --- |
| CPU | ARM CA53 (2GHz, Dual-core) |
| Chip model | MT7987BV+MT7990AN+MT7976CN+AN8855H |
| Memory | NAND FLASH 1Gbit(128MB)/DDR3 4Gbit（512MB） |
| WiFi Chip | MT7990AN+MT7976CN |
| WiFi eFEM | N/A (iPAiLNA) |
| Network Protocol | IPv4/IPv6 Dual Stack NAT/PAT/ALG Port mapping/DMZ/Virtual server UPnP IGD with NAT traversal capability Static routing/dynamic routing Dynamic DNS relay DHCP client/server IGMP and MLD snooping/proxy |
| Interface | WAN: 1* 10/100/1000Mbps/2500Mbps auto MDI/MDI-X RJ-45 ports LAN: 3* 10/100/1000Mbps auto MDI/MDI-X RJ-45 ports USB3.0 *1 |
| Buttons | WPS/WiFi ，Reset，Power on/off |
| LED Indicators | Power, Internet, LAN1, LAN2, LAN3, WAN, 2.4G WLAN, 5G WLAN, WPS, USB |
| WiFi Rate | 2.4G:802.11b/g/n/ax/be up to 689Mbps 5G:802.11/a/n/ac/ax/be up to 2882Mbps |
| Frequency Band | 2.4 GHz frequency band 20/40MHz 5 GHz frequency band 20/40/80/160MHz |
| Antennas | 2.4G：2*5dBi External antennas 5G： 3*5dBi External antennas |
| Power Supply | DC 12V 1.5A |

| Product Size | 180x120x42.2 mm (length x width x height) |
| --- | --- |
| Work Environment | Working temperature: 0°C—40°C Working humidity: 5—95% |
| WAN Connection | Support DHCP, PPPoE and Static IP address three internet access models |
| Wireless Function | WiFi basic function, Guest network, Intelligent networking, Wireless Repeater， Easy mesh |
| Wireless Encryption | WPA/WPA2, WPA2, WPA3 |
| Management Function | WEB, Telnet/CLI,TR069 |
| System Maintenance | Device management, Configuration management, Upgrade management, Log function, Status statistics, Restart/reset |
| Advanced Function | Parental control, Virtual server, DDNS, VPN, IPV6, UPnP |
| Certification | Meet CE certification |