AZ797M

3GE AC1200 Mesh Router

Descriptions

AZ797M is a concurrent dual band AC1200 mesh router, which has been upgraded to the latest 802.11ac standard

technology while maintaining compatibility with IEEE 802.11b/g/n and IEEE 802.11a networks. It’s a SOHO-class

network security solution for family home, broadband communications and small SOHO.

Features

●   1x 10/100/1000BASE-T Ethernet RJ45 WAN Ports ;

●   2x 10/100/1000BASE-T Ethernet RJ45 LAN Ports ;

●   Support 802.11n 2T2R speed up to 300Mbps ;

●   Support 802.11ac 2T2R speed up to 867Mbps ;

●   Four Antennas for good coverage;

Specifications

| AZ797M Specifications |  |
| --- | --- |
| CPU | Mips (1000MHz，single core) |
| Chip model | RTL8197FH-VG4 +RTL8367RB+ RTL8812FR |
| Memory | 64Mbit(8MB)/DDR2 512Mbit（64MB） |
| WiFi Chip | 2.4G：SOC 5G：RTL8812FR |
| Interface | WAN: 1* 10/100/1000Mbps auto MDI/MDI-X RJ-45 ports LAN: 2* 10/100/1000Mbps auto MDI/MDI-X RJ-45 ports |
| Buttons | Reset |
| LED Indicators | Power, WAN, WiFi, ETH |
| WiFi Rate | 2.4G:802.11b/g/n@300Mbps 5G:802.11/a/n/ac@867Mbps |
| Frequency Band | 2.4 GHz frequency band 40MHz and 5 GHz frequency band 80MHz |
| Antennas | 2.4G：2*3dBi Internal antennas 5G：2*3dBi Internal antennas |
| Power Supply | DC 12V 1A |
| Product Size | 90*90*65mm(length X width X height) |
| Work Environment | Working temperature: 0°C—40°C Working humidity: 5—95% |
| WAN Connection | Support DHCP, PPPoE and Static IP address three internet access models, Support WAN automatic detection, MAC Clone |
| Wireless Function | WiFi5 basic function, Guest network, Intelligent networking, Wireless Repeater， Easy mesh R2 |
| Wireless Encryption | Open system and shared key；WPA-PSK，WPA2-PSK，WPA/WPA2-PSK；AES，TKIP en-cryption； |
| Management Function | WEB, Telnet/CLI |
| System Maintenance | Device management, Configuration management, Upgrade management, Log function, Status statistics, Restart/reset |
| Advanced Function | Parental control, Virtual server, DDNS, VPN, IPV6, UPnP ARP、ICMP、NAT、ALG，Firewall |
| Certification | Meet CE certification |