AZ846

N300 4G CPE Router

Descriptions

AZ846 is a N300 CPE with 4G modem, which maintaining compatibility with IEEE 802.11b/g/n networks.

It’s a SOHO-class network security solution for family home, broadband communications and small SOHO.

Features

●   1x 10/100MBASE-T Ethernet RJ45 WAN Ports;

●   3x 10/100MBASE-T Ethernet RJ45 LAN Ports;

●   Support Wi-Fi 2.4G 802.11N 2T2R, speed up to 300Mbps;

●   Support LTE CAT4 with uplink speed up to 50Mbps, and downlink speed up to 150Mbps;;

Specifications

| AZ846 Specifications |  |
| --- | --- |
| Chip model | MT7628NN+CAT4 LTE Moudle |
| Memory | FLASH 16MB/DDR2 64MB |
| WiFi Chip | Built in MT7628NN |
| Cellular Frequency | LTE-FDD: B1/ 3/ 5/7/ 8/20/28 LTE-TDD: B38/40/41 WCDMA: B1/5/8 |
| WiFi rate | 802.11b/g/n Wireless LAN 300Mbps |
| Interface | LAN: 3* 10/100Mbps auto MDI/MDI-X RJ-45 ports LAN/WAN: 1* 10/100Mbps auto MDI/MDI-X RJ-45 ports Sim slot：1*nano sim card |
| Frequency Band | 2.4 GHz frequency band 40MHz |
| Antennas | WIFI 2.4G: 2*5dBi External antennas LTE: 2*3dBi External antennas |
| Buttons | Reset |
| LED Indicators | Power NET LAN1 LAN2 LAN3 WAN WIFI LTE |
| Power Supply | DC 12V 1A |
| Product Size | 180X110X35 mm (length X width X height) |
| Work Environment | Working temperature: 0°C—40°C；Working humidity: 10—90% |
| WAN Connection | Support DHCP, PPPoE and Static IP address three internet access models, |
| Wireless Function | WiFi basic function, WiFi advanced function, Guest network, |
| Wireless Encryption | WPA/WPA2, WPA2 |
| Management Function | WEB |
| System Maintenance | Device management, Configuration management, Upgrade management, Log function, Status statistics, Restart/reset |
| Advanced Function | Firewall, Virtual server,IPV6, UPnP |
| Certification | Meet CE certification |