AZ848

AX3000 CAT6 4G CPE Router

Descriptions

AZ848 is a dual band concurrent AX3000 CPE with cat6 4G modem, which has been upgraded to the latest 802.11ax

standard technology while maintaining compatibility with IEEE 802.11b/g/n and IEEE 802.11a networks. It’s a SOHO-class

network security solution for family home, broadband communications and small SOHO.

Features

●   1x 10/100/1000BASE-T Ethernet RJ45 WAN Ports;

●   3x 10/100/1000BASE-T Ethernet RJ45 LAN Ports;

●   Support 802.11ax 2T2R speed up to 574Mbps;

●   Support 802.11ax 2T3R speed up to 2402Mbps;

●   Support LTE CAT6 with uplink speed up to 50Mbps, and downlink speed up to 300Mbps;

●   Support one USB2.0 ports;

Specifications

| AZ848 Specifications |  |
| --- | --- |
| Chip model | MT7981B +MT7976CN+MT7531AE+CAT6 LTE Moudle (EG060WEAAA) |
| Memory | NAND FLASH 1Gbit(128MB)/DDR4 4Gbit（512MB） |
| WiFi Chip | 2.4G&5G: MT7976CN |
| Cellular Frequency | LTE-FDD: B1/B3/B5/B7/B8/B20/B28 LTE-TDD: B38/B40/B41 WCDMA: B1/5/8 |
| WiFi rate | 2.4G:802.11b/g/n/ax@574Mbps 5G:802.11/a/n/ac/ax@2402Mbps |
| Interface | WAN: 1* 10/100/1000Mbps auto MDI/MDI-X RJ-45 ports LAN: 3* 10/100/1000Mbps auto MDI/MDI-X RJ-45 ports 1*nano sim card 1*USB2.0 1*DC JACK |
| Frequency Band | 2.4 GHz frequency band 40MHz and 5 GHz frequency band 160MHz |
| Antennas | WIFI : 1*3dBi External dual-band omni antennas 1*5dBi External 2.4G omni antennas 2*5dBi External 5G omni antennas LTE: 2*3dBi External omni antennas |
| Buttons | WPS，Reset，Power on/off |
| LED Indicators | Power, NET, LAN3, LAN2, LAN1, WAN, 2.4G, 5G, WPS, LTE |
| Power Supply | DC 12V 2A |
| Product Size | 180X120X42.2 mm (length X width X height) |
| Work Environment | Working temperature: 0°C—40°C；Working humidity: 10—90% |
| WAN Connection | Support DHCP, PPPoE and Static IP address three internet access models |
| Wireless Function | WiFi basic function, WiFi advanced function, Guest network |
| Wireless Encryption | WPA/WPA2, WPA2 |
| Management Function | WEB |
| System Maintenance | Device management, Configuration management, Upgrade management, Log function, Status statistics, Restart/reset |
| Advanced Function | Firewall, Virtual server,IPV6, UPnP |
| Certification | Meet CE certification |